# S1 Appendix

To simulate blood flow in the patient-specific coronary artery tree models, this study used FLUENTTM to solve continuity and Navier-Stokes equations as shown in Eqs. A1 and A2 respectively.

(A1)

(A2)

where xj is the location in Cartesian coordinates, uj (or ui) is the Cartesian component of velocity, P represents the static pressure; and ρ and μ were set as 1,060 kg/m3 and 4.5×10-3 Pa·s respectively to represent the density and dynamic viscosity of blood in large epicardial arteries [1,2].

Pressure and resistance boundary conditions were specified at the inlet and outlets of each model respectively to mimic physiological conditions. To assign the inlet total pressure, patient-specific systolic and diastolic brachial pressure was used to calculate the mean brachial pressure and match the mean aortic pressure.

A key element in assigning coronary resistance values of the outlets is to prescribe reference pressure (P0). Physiologic studies report that coronary pressure flow lines are concave to the axis of flow at lower pressures but straight at physiological pressures [3,4]. The zero flow pressure intercept at the physiologic pressure range (i.e., P0 in this study) exceeds coronary venous or left ventricular diastolic pressure by five to ten-fold [5]. Therefore, we propose a novel method in this study to determine the reference pressure P0, and therefore Ri through an iterative procedure. The details are provided as following.

Briefly, the total resistance RtotalRinlet at resting condition is defined by Eq. A3

Rtotal=(P-P0)/Qtotal                                                 (A3)

where Pinlet represents the mean aortic pressure, estimated from the mean branchial artery pressure. The total coronary flow at resting can be estimated from the myocardial mass [6,7]. Accordingly Qinlet can be determined from CT images.

At each coronary outlet,

Pi=P0+RiQi                                                              (A4)

The correlation between the resistance of downstream vasculature of each coronary branch (Ri) and that of the coronary tree (Rinlet) can be estimated from the scaling law [8],

Ri=NiRinlet                                                           (A5)

for non-stenotic vessels. Because the resistance of downstream vasculature was determined by the inherent anatomy rather than by upstream structures [9], the resistance of vessels with stenosis satisfied the same relationship as Eq. A5.

From A3 and A5, Ri can have the following form:

Ri=NiPinlet-P0Qinlet=Pi-P0Qi                                                    (A6)

By the mass conservation law, total outflow was equal to inflow (Eq. A7),

i=1NQi=Qinlet                                                         (A7)

From Eqs. A4, A5 and A7, it can be shown that P0 is given by:

P0=Pinlet-i=1NPi-P0Ni                                                  (A8)

Ri=NiPinlet-P0Qinlet            At hyperemia, coronary microcirculation was assumed to have predictable response to adenosine and the resistance was assumed to be reduced to K times the resting values (where K=0.21), a value within the physiological range measured by Wilson et al. [10].  From Eq. (A6), the hyperemic resistance is given by:

Ri,hypermia=KRi=KNiPinlet-P0Qinlet                                   (A9)

In order to ensure smooth convergence during the numerical iterations of CFD, P0 and Ri were initialized as 20 mmHg and 1.0e+8 Pa·s/m3, respectively; and then updated with a under-relaxation scheme as formulated in Eqs. A10 and A11 until the total outflow from all the outlets matched the inflow rate at hyperemia.

P0, new=1-αP0,old+α (Pinlet-i=1NPi-P0Ni)                               (A10)

Ri,hypermia,new=1-αRi,hypermia,old+α(KNiPinlet-P0Qinlet)                        (A11)

Here, α was under-relaxation factor. P0,old and Ri,hyperemia, old represented the reference pressure and resistance in the last iteration, while P0,new and Ri,hyperemia,new represented their values in the next iteration. As minimal microvascular resistance was found to be independent of epicardial stenosis severity [11], hyperemic microcirculatory resistance distal to a stenosis was assumed to be the same as that for coronary arteries free of stenosis [12] as specified in Eq. A11.

With these novel iterative boundary conditions, the computational burden can be reduced in contrast to solving the complex lumped-parameter heart and coronary models described in Ref. 16. The calculated value of P0 was 38.6 ± 9.5 mmHg for the 21 patients investigated in this study, which was close to 37.9 ± 9.8 mmHg measured by Dole et al. [5] for 10 patients and 36±9 mmHg measured by Nanto et al. [13] for 15 subjects respectively at vasodilated state. A no-slip boundary condition was imposed on the vessel wall.

References

1. 	Huo Y, Wischgoll T, Kassab GS. Flow patterns in three-dimensional porcine epicardial coronary arterial tree. Am J Physiol Heart Circ Physiol. 2007;293: H2959–H2970. doi:10.1152/ajpheart.00586.2007

2. 	Zhang JM, Chua LP, Ghista DN, Yu SCM, Tan YS. Numerical investigation and identification of susceptible sites of atherosclerotic lesion formation in a complete coronary artery bypass model. Med Biol Eng Comput. 2008;46: 689–699. doi:10.1007/s11517-008-0320-4

3. 	Klocke FJ, Weinstein IR, Klocke JF, Ellis AK, Kraus DR, Mates RE, et al. Zero-flow pressures and pressure-flow relationships during single long diastoles in the canine coronary bed before and during maximum vasodilation. Limited influence of capacitive effects. J Clin Invest. 1981;68: 970–980. doi:10.1172/JCI110351

4. 	Spaan JAE, Piek JJ, Hoffman JIE, Siebes M. Physiological basis of clinically used coronary hemodynamic indices. Circulation. 2006. pp. 446–455. doi:10.1161/CIRCULATIONAHA.105.587196

5. 	Dole WP, Richards KL, Hartley CJ, Alexander GM, Campbell AB, Bishop VS. Diastolic coronary artery pressure-flow velocity relationships in conscious man. Cardiovasc Res. 1984;18(9): 548–554.

6. 	Hamada M, Kuwahara T, Shigematsu Y, Kodama K, Hara Y, Hashida H, et al. Relation between coronary blood flow and left ventricular mass in hypertension: noninvasive quantification of coronary blood flow by thallium-201 myocardial scintigraphy. Hypertens Res. 1998;21: 227–234.

7. 	Wieneke H, von Birgelen C, Haude M, Eggebrecht H, Möhlenkamp S, Schmermund A, et al. Determinants of coronary blood flow in humans: quantification by intracoronary Doppler and ultrasound. Journal of Applied Physiology. 2005. doi:10.1152/japplphysiol.00724.2004

8. 	Zhou Y, Kassab GS, Molloi S. On the design of the coronary arterial tree: a generalization of Murray’s law. Phys Med Biol. 1999;44: 2929–2945. doi:10.1088/0031-9155/44/12/306

9. 	Kim H, Vignon-Clementel I, Coogan J, Figueroa C, Jansen K, Taylor C. Patient-specific modeling of blood flow and pressure in human coronary arteries. Ann Biomed Eng. Springer Netherlands; 2010;38: 3195–3209. doi:10.1007/s10439-010-0083-6

10. 	Wilson RF, Wyche K, Christensen BV, Zimmer S LD. Effects of adenosine on human coronary arterial circulation. Circulation. 1990;82: 1595–1606. doi:10.1161/01.CIR.82.5.1595

11. 	Aarnoudse W, Fearon WF, Manoharan G, Geven M, Van De Vosse F, Rutten M, et al. Epicardial stenosis severity does not affect minimal microcirculatory resistance. Circulation. 2004;110: 2137–2142. doi:10.1161/01.CIR.0000143893.18451.0E

12. 	Taylor CA, Fonte TA, Min JK, City R, Angeles L. Computational fluid dynamics applied to cardiac computed tomography for noninvasive quantification of fractional flow reserve: scientific basis. J Am Coll Cardiol. 2013;61: 2233–2241. doi:10.1016/j.jacc.2012.11.083

13. 	Nanto S, Masuyama T, Takano Y. Determination of coronary zero flow pressure by analysis of the baseline pressure – flow relationship in humans. Jpn Circ J. 2001;65 (9): 793–796.